# Packet Tracer - Skills Integration Challenge

| 1.3.1.3 Packet Tracer - Skills Integration Challenge.pka | Connect Console cable from User-01 (RS 232) to ASw-1 (Console) ASw-1 -> Startup Config Switch>enable Switch(config)#copy running-config startup-config ASw-1 -> Ports -> Vlan1 -> IP Address/Subnet Mask Switch#configure terminal Switch(config)#interface vlan 1 Switch(config-if)#ip address 10.10.10.100 255.255.255.0 ASw-1 -> Ports -> Vlan1 -> Port Status Switch(config-if)#no shutdown Switch(config-if)#exit ASw-1 -> Banner MOTD Switch(config)#banner motd "Warning, you must be admin" ASw-1 -> Console Line -> Password Switch(config)#line console 0 Switch(config-line)#password xAw6k Switch(config-line)#exit ASw-1 -> Enable Secret Switch(config)#enable secret 6EBUp ASw-1 -> Host Name Switch(config)#hostname ASw-1 ASw-1 -> Service Password Encryption ASw-1(config)#service password-encryption ASw-1 -> VTY Lines -> VTY Line 0 -> Password ASw-1(config)#line vty 0 ASw-1(config-line)#password xAw6k ASw-1(config-line)#exit Connect Console cable from User-02 (RS 232) to ASw-2 (Console) Do all ASw-1 steps (except hostname = ASw-2 and Vlan1 = ip address 10.10.10.150 255.255.255.0) Open “IP Configuration” for User-01 IP Address: 10.10.10.4 Subnet Mask: 255.255.255.0 Open “IP Configuration” for User-02 IP Address: 10.10.10.5 Subnet Mask: 255.255.255.0 |
| --- | --- |
| 2.3.1.2 Packet Tracer - Skills Integration Challenge.pka | Click Switch -> CLI S1 -> Banner MOTD Switch>enable Switch#configure terminal Switch(config)#banner motd "Warning, you must be admin" S1 -> Console Line -> Password & Login Switch(config)#line console 0 Switch(config-line)#password cisco Switch(config-line)#login Switch(config-line)#exit S1 -> Enable Secret Switch(config)#enable secret class S1 -> Host Name Switch(config)#hostname S1 S1 -> IP Domain Name S1(config)#ip domain-name cisco.com S1 -> Ports -> FastEthernet0/x -> Port Status S1(config)#interface range FastEthernet0/1-24 S1(config-if-range)#shutdown S1(config-if-range)#exit S1 -> Ports -> FastEthernet0/1 & 2 -> Port Security -> Enabled & Port Security Violation S1(config-if-range)#interface range FastEthernet0/1-2 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport port-security S1 -> Ports -> FastEthernet0/1 &2 -> Port Security -> Maximum Static MACs (config-if-range)#switchport port-security maximum 2 S1 -> Ports -> FastEthernet0/6 -> Port Security -> Sticky Enabled S1(config-if-range)#switchport port-security mac-address sticky S1(config-if-range)#exit S1 -> Ports -> GigabitEthernet0/1 & 2 -> Port Status S1(config)#interface range GigabitEthernet0/1-2 S1(config-if-range)#shutdown S1(config-if-range)#exit S1 -> Ports -> Vlan 1 -> IP Address & Subnet Mask S1(config)#interface vlan 1 S1(config-if)#ip address 10.10.10.2 255.255.255.0 S1 -> Ports -> Vlan 1 -> Port Status S1(config-if)#no shutdown S1(config-if)#exit S1 -> Service Password Encryption (config)# service password-encryption --- Generate RSA 768 bits token S1(config)#crypto key generate rsa How many bits in the modulus [512]: 768 % Generating 768 bit RSA keys, keys will be non-exportable...[OK] --- S1 -> SSH Server -> SSH Version S1(config)#ip ssh version 2 S1 -> Usernames -> Username (hit “secret password”) S1(config)#username admin secret ccna S1 -> VTY Lines -> VTY Line 0 -> Login S1(config)#line vty 0 S1(config-line)#login local S1 -> VTY Lines -> VTY Line 0 -> Transport Input S1(config-line)#transport input ssh S1(config-line)#exit Restart only two ports S1(config)#interface range FastEthernet0/1-2 S1(config-if-range)#no shutdown Open PC1 and PC2 -> Desktop -> IP Configuration Set IP Address: 10.10.10.10 or 10.10.10.11 Subnet Mask: 255.255.255.0 Gateway: 10.10.10.1 Open Command Prompt Ping: 10.10.10.2 (Switch) Ping 10.10.10.10 (PC1) Ping 10.10.10.11 (PC2) |
| 2.5.1.2_Packet_Tracer_-_Configure_Cisco_Routers for_Syslog,_NTP,_and_SSH_Operations[1].pka | R1 -> Logging -> Service timestamp log R1>enable Password: (ciscoenpa55) R1#configure terminal R1(config)#service timestamps log datetime msec (Check on the Dashboard IP Address of “NTP Server”) R1 -> NTP Client -> NTP Server Information -> IP R1(config)#ntp server 192.168.1.5 R1 -> NTP Client -> NTP Server Information -> Update Calendar R1(config)#ntp update-calendar (Verify: R1(config)#do show ntp status) (Verify: R1(config)#do show clock) R1 -> SYSLOG Client -> Server Addresses R1(config)#logging host 192.168.1.6 (Verify: R1(config)#do show logging) Click on “Syslog Server” -> Services -> SYSLOG Type in R1 -> CLI : R1(config)#exit Do the same steps in R2 and R3 routers --- R3 -> IP Domain Name R3(config)#ip domain-name ccnasecurity.com --- Generate RSA 1024 bits token (see notes) S1(config)#crypto key generate rsa How many bits in the modulus [512]: 1024 % Generating 768 bit RSA keys, keys will be non-exportable...[OK] Verify: R3(config)#do show ip ssh) --- R3 -> SSH Server -> SSH Authentication Retries R3(config)#ip ssh authentication-retries 2 R3 -> SSH Server -> SSH Timeout R3(config)#ip ssh time-out 90 R3 -> SSH Server -> SSH Version R3(config)#ip ssh version 2 R3 -> User Names -> Username (Verify: R3(config)#username SSHadmin privilege ? <0-15> User privilege level) R3(config)#username SSHadmin privilege 15 secret ciscosshpa55 R3 -> VTY Lines -> VTY Line 0 to 4 -> Login R3(config)#line vty 0 4 R3(config-line)#login local R3 -> VTY Lines -> VTY Line 0 to 4 -> Transport Input R3(config-line)#transport input ssh R3(config-line)#exit Open PC-C -> Desktop -> Command Prompt C:\>telnet 192.168.3.1 Trying 192.168.3.1 ...Open [Connection to 192.168.3.1 closed by foreign host] C:\>ssh -l SSHadmin 192.168.3.1 Password: (ciscosshpa55) R3#exit |
| 3.3.3.3_Packet_Tracer_-_Explore_a_Network[1].pka | It’s simulator demo from “Sales” PC |
| 3.4.1.2 Skills Integration Challenge.pka | Only S3 CLI is available we use connect console cables. Open S3 CLI (some conflict Gig0/2 connection, temporary shutdown) S3>enable S3#configure terminal S3(config)# S3(config)#interface GigabitEthernet0/2 S3(config-if)#shutdown S3(config-if)#exit S3 -> Banner MOTD S3(config)#banner motd "Authorized Access Only!!" S3 -> Console Line -> Password S3(config)#line console 0 S3(config-line)#password letmein S3(config-line)#exit S3 -> Default Gateway S3(config)# ip default-gateway 172.31.88.1 S3 -> Enable Secret S3(config)#enable secret itsasecret S3 -> Ports -> FastEthernet0/11 -> Access VLAN S3(config)#interface range FastEthernet0/7-12 S3(config-if-range)#switchport access vlan 10 S3(config-if-range)#exit S3 -> Ports -> FastEthernet0/18 -> Access VLAN S3(config)#interface range FastEthernet0/13-20 S3(config-if-range)#switchport access vlan 20 S3(config-if-range)#exit S3 -> Ports -> FastEthernet0/6 -> Access VLAN S3(config)#interface range FastEthernet0/1-6 S3(config-if-range)#switchport access vlan 30 S3 -> Ports -> FastEthernet0/6 -> Port Security -> Enabled S3(config-if-range)#switchport mode access S3(config-if-range)#switchport port-security S3 -> Ports -> FastEthernet0/6 -> Port Security -> Maximum Static MACs (config-if-range)#switchport port-security maximum 2 S3 -> Ports -> FastEthernet0/6 -> Port Security -> Port Security Violation S3(config-if-range)#switchport port-security violation restrict S3 -> Ports -> FastEthernet0/6 -> Port Security -> Sticky Enabled S3(config-if-range)#switchport port-security mac-address sticky S3(config-if-range)#exit S3 -> Ports -> FastEthernet0/4 & 0/14 & 0/24 -> Port Status S3(config)#interface range FastEthernet0/1-24 S3(config-if-range)#shutdown S3(config-if-range)#exit Check Dashboard - enable F0/6 & F0/11 & F0/18 S3(config)#interface FastEthernet0/6 S3(config-if)#no shutdown S3(config-if)#exit … S3 -> Ports -> GigabitEthernet0/2 -> Native VLAN S3(config)#interface GigabitEthernet0/2 S3(config-if)#switchport trunk native vlan 99 S3 -> Ports -> GigabitEthernet0/2 -> Nonegotiate S3(config-if)# switchport nonegotiate S3 -> Ports -> GigabitEthernet0/2 -> Port Mode S3(config-if)#switchport mode trunk S3 -> Ports -> GigabitEthernet0/2 -> Trunk VLANs -> 10 & 20 & 30 & 88 & 99 S3(config-if)#switchport trunk allowed vlan 10 etc. or S3(config-if)#switchport trunk allowed vlan all S3(config-if)#exit S3 -> Ports -> Vlan88 -> Port Status S3(config)#interface vlan 88 S3(config-if)#no shutdown S3 -> Ports -> Vlan88 -> IP Address / Subnet Mask S3(config-if)#ip address 172.31.88.4 255.255.255.0 S3(config-if)#exit S3 -> Service Password Encryption (config)# service password-encryption S3 -> VLANS -> VLAN 10 -> VLAN Name S3(config)#vlan 10 S3(config-vlan)#name Sales S3(config-vlan)#exit (Assign name for other VLANs) S3 -> VTY Lines -> VTY Line 0 -> Password S3(config)#line vty 0 S3(config-line)#password c1$c0 S3(config-line)#exit Restart GigabitEthernet0/2 S3(config)#interface GigabitEthernet0/2 S3(config-if)#no shutdown |
| 3.6.1.2_Packet_Tracer_-_Configure_AAA_Authentication_on_Cisco_Routers[1].pka | R1 -> AAA -> New-model R1>enable Password: (ciscoenpa55) R1#configure terminal R1(config)#aaa new-model R1 -> AAA -> Authentication -> Authen Command 1 R1(config)#aaa authentication login TELNET-LOGIN local R1 -> AAA -> Authentication -> Authen Command 2 R1(config)#aaa authentication login default local R1 -> Console Line -> AAA Method List Name R1(config-line)#line console 0 R1(config-line)#login authentication default R1(config-line)#exit R1 -> User Names -> Username username Admin1 secret admin1pa55 R1 -> VTY Lines -> VTY Line 0 to 4 -> AAA Method List Name R1(config)#line vty 0 4 R1(config-line)#login authentication TELNET-LOGIN R1(config-line)#exit Router R2 R2 -> AAA -> New-model R2>enable Password: (ciscoenpa55) R2#configure terminal R2(config)#aaa new-model R2 -> AAA -> Authentication -> Authen Command 1 R2(config)#aaa authentication login default group tacacs+ local R2 -> Console Line -> AAA Method List Name R2(config-line)#login authentication default R2(config-line)#exit R2 -> TACACS Client -> TACAS Server Hosts -> 0 tacacs-server host 192.168.2.2 R2 -> TACACS Client -> TACAS server key R2(config)#tacacs-server key tacacspa55 R2 -> User Names -> Username R2(config)#username Admin2 secret admin2pa55 Router R3 R3 -> AAA -> New-model R3>enable Password: (ciscoenpa55) R3#configure terminal R3(config)#aaa new-model R3 -> AAA -> Authentication -> Authen Command 1 R3(config)#aaa authentication login default group radius local R3 -> Console Line -> AAA Method List Name R3(config-line)#login authentication default R3(config-line)#exit R3 -> RADIUS Client -> RADIUS Server Hosts -> 0 R3(config)#radius-server host 192.168.3.2 R3 -> RADIUS Client -> RADIUS server key R3(config)# radius-server key radiuspa55 R3 -> User Names -> Username R3(config)#username Admin3 secret admin3pa55 |
| 5.4.1.2 Packet Tracer - Skills Integration Challenge.pka | R1(config)#interface G0/0 R1(config-if)#ip address 172.17.25.2 255.255.255.252 R1(config-if)#no shutdown R1(config-if)#exit R1 -> Ports -> GigabitEthernet0/1.10 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.10 R1(config-subif)#encapsulation dot1Q 10 R1 -> Ports -> GigabitEthernet0/1.10 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.10.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1.20 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.20 R1(config-subif)#encapsulation dot1Q 20 R1 -> Ports -> GigabitEthernet0/1.20 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.20.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1.30 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.30 R1(config-subif)#encapsulation dot1Q 30 R1 -> Ports -> GigabitEthernet0/1.30 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.30.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1.88 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.88 R1(config-subif)#encapsulation dot1Q 88 R1 -> Ports -> GigabitEthernet0/1.88 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.88.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1.99 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.99 R1(config-subif)#encapsulation dot1Q 99 R1 -> Ports -> GigabitEthernet0/1.99 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.99.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1 -> Port Status R1(config-subif)#interface G0/1 R1(config-if)#no shutdown Switch S1 S1 -> Default Gateway S1>enable S1#configure terminal S1(config)#ip default-gateway 172.17.99.1 S1 -> Ports -> FastEthernet0/10 -> Access VLAN S1(config)#interface range F0/6-10 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vlan 30 S1(config-if-range)#exit S1 -> Ports -> FastEthernet0/11 -> Access VLAN S1(config)#interface range F0/11-17 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vlan 10 S1(config-if-range)#exit S1 -> Ports -> FastEthernet0/18 -> Access VLAN S1(config)#interface range F0/18-24 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vlan 20 S1(config-if-range)#exit S1 -> Vlan99 -> Post Status S1(config)#interface vlan 99 S1 -> Vlan99 -> IP Address & Subnet Mask S1(config-if)#ip address 172.17.99.10 255.255.255.0 S1(config-if)#exit S1 -> VLANS -> VLAN 10 -> VLAN Name S1(config)#vlan 10 S1(config-vlan)#name Faculty/Staff S1 -> VLANS -> VLAN 20 -> VLAN Name S1(config)#vlan 20 S1(config-vlan)#name Students S1 -> VLANS -> VLAN 30 -> VLAN Name S1(config)#vlan 30 S1(config-vlan)#name Guest(Default) S1(config-vlan)#exit S1 -> VLANS -> VLAN 88 -> VLAN Name S1(config)#vlan 88 S1(config-vlan)#name Native S1(config-vlan)#exit S1 -> VLANS -> VLAN 99 -> VLAN Name S1(config)#vlan 99 S1(config-vlan)#name Management S1(config-vlan)#exit S1 -> Ports -> GigabitEthernet0/1 S1(config)#interface G0/1 S1(config-if)#switchport trunk native vlan 88 S1(config-if)#exit Additional Requirements to shutdown all not assigned ports S1(config)#ip default-gateway 172.17.99.1 S1(config)#interface range F0/1-5, G0/2 S1(config-if-range)#shutdown S1(config-if-range)#exit |
| 6.6.1.2 Packet Tracer - Skills Integration Challenge.pka |  |